ORIENTATION SYSTEM

Architecture & Implementation Specification

ESP32 + MPU6500 + TLV493D → Jetson Orin Nano

Rev 1.0  |  Build-from-scratch reference

# 1. System Overview

The system reads attitude from two sensors on the ESP32, runs a Madgwick AHRS filter at 100 Hz, and streams calibrated Euler angles to a Jetson Orin Nano over UART at 115200 baud. All heavy computation runs on the ESP32. The host is a thin consumer.

## 1.1  Hardware Summary

| Component | Part | Interface | Role |
| --- | --- | --- | --- |
| IMU | MPU6500 | I2C @ 400 kHz | Accel + Gyro |
| Magnetometer | TLV493D | I2C @ 400 kHz (shared bus) | Magnetic heading |
| MCU | ESP32 (38-pin) | USB-Serial to host | Filter + comms |
| Host | Jetson Orin Nano | /dev/ttyUSB0 | Python consumer |

## 1.2  Coordinate Frame (ENU, fixed for the entire codebase)

Both sensors are physically mounted so their chip axes align to ENU. No software axis remapping is needed anywhere in the pipeline.

| Axis | World direction | Sensor reading | Notes |
| --- | --- | --- | --- |
| X | East | Positive = East | Both chips verified |
| Y | North | Positive = North | Yaw reference axis |
| Z | Up (Zenith) | Positive = Up | Gravity reference |

## 1.3  Euler Output Convention

| Angle | Range | Zero point | Positive direction |
| --- | --- | --- | --- |
| Yaw | 0 – 360 deg | North (Y axis) | Clockwise (compass bearing convention) |
| Pitch | -90 – +90 deg | Horizontal | Nose up toward zenith |
| Roll | -180 – +180 deg | Level | Right side down toward East |

Rotation order: ZYX. Internal computation is always quaternion (float32). Euler angles are produced only at the output stage, once per packet.

## 1.4  Magnetic Declination

Declination for Bengaluru: -0.8 degrees (hardcoded). Define it as a named constant. Document that this value is site-specific and must be updated if the device is relocated.

static constexpr float kMagDeclinationDeg = -0.8f;  // Bengaluru, India

# 2. Firmware Architecture

The firmware is a single PlatformIO project targeting the ESP32 Arduino framework. Every module is a .cpp/.h pair with a clear, single responsibility. main.cpp contains only setup() and loop(). All state lives inside the Application class.

## 2.1  Directory Layout

firmware/

platformio.ini

src/

main.cpp                   # setup() and loop() only

core/

Config.h                 # all compile-time constants

SystemState.h/.cpp        # state machine

Error.h/.cpp              # ErrorCode enum + toString

Clock.h                   # millis() wrapper

EventQueue.h/.cpp         # fixed-size event ring

drivers/

HalI2c.h/.cpp             # Wire wrappers (read/write/probe)

DrvMpu6500.h/.cpp         # raw accel + gyro reads

DrvTlv493d.h/.cpp         # raw mag reads + frame counter

calibration/

NvsCal.h/.cpp             # NVS blob persist/load/clear

CalMag.h/.cpp             # ellipsoid fit, sphere coverage

CalImu.h/.cpp             # gyro bias learner, accel bias

fusion/

Madgwick.h/.cpp           # AHRS filter (quaternion, ENU)

comms/

UartTransport.h/.cpp      # ring-buffered RX, line timeout

CommandParser.h/.cpp      # ASCII command tokenizer

PacketWriter.h/.cpp       # 17-byte binary output + CRC16

app/

Application.h/.cpp        # top-level wiring, main loop

## 2.2  Build Flags (platformio.ini)

[env:esp32dev]

platform         = espressif32

board            = esp32dev

framework        = arduino

monitor_speed    = 115200

upload_speed     = 921600

build_flags      = -std=gnu++17 -Wall -Wextra -Werror

-Wconversion -Wshadow -Wdouble-promotion

Treat all warnings as errors (-Werror). This prevents silent float-to-double promotions and conversion bugs which are common in sensor math code. Do not remove -Wdouble-promotion; it will catch any accidental use of double arithmetic in the filter loop.

# 3. Core Layer

## 3.1  Config.h

All numeric constants live here and nowhere else. No magic numbers anywhere in the codebase. Every constant is constexpr in the core::config namespace.

| Constant | Value | Purpose |
| --- | --- | --- |
| kSerialBaud | 115200 | UART baud rate |
| kImuUpdateHz | 200 | MPU6500 read rate (5 ms period) |
| kFilterHz | 100 | Madgwick update and packet output rate |
| kBetaStill | 0.033f | Madgwick beta when static |
| kBetaMove | 0.1f | Madgwick beta during motion |
| kGyroStillRingN | 200 | Sample window for stillness detection |
| kGyroStillThreshRs | 0.01f | Max gyro std-dev (rad/s) for still condition |
| kAutoRecalIntervalMs | 1800000 | 30 min between auto-recal triggers |
| kMagSphereBins | 72 | Coverage bins (12 azimuth x 6 elevation) |
| kMagMinSamples | 200 | Min samples before ellipsoid fit |
| kMagMinCoveragePct | 60 | Min sphere coverage % before fit allowed |
| kMagDeclinationDeg | -0.8f | Bengaluru magnetic declination |
| kCalMagicNumber | 0xCA11BA11 | NVS blob validity marker |
| kCalVersion | 1 | Increment when CalBlob layout changes |
| kReadyTimeoutMs | 10000 | Host waits this long for READY after reset |

## 3.2  SystemState

A guarded state machine. transitionTo() validates allowed transitions before changing state. Any illegal transition returns an error rather than silently corrupting state.

States: BOOT → SELF_TEST → IDLE → CALIBRATING → READY → ERROR

| State | Meaning |
| --- | --- |
| BOOT | Power-on default, pre-init |
| SELF_TEST | Sensor WHO_AM_I and sanity checks running |
| IDLE | Sensors up, no valid calibration loaded |
| CALIBRATING | Active calibration collection in progress |
| READY | Valid cal loaded, filter converged, packets streaming |
| ERROR | Unrecoverable fault; CMD:RESET required |

# 4. Sensor Drivers

## 4.1  HalI2c.h/.cpp

Thin wrappers around Wire. Keep driver code readable by hiding the raw Wire calls here. All functions return bool. On failure they return false and do not assert or panic.

| Function | Behaviour |
| --- | --- |
| i2cProbe(addr) | beginTransmission + endTransmission, returns true if ACK |
| i2cReadReg8(addr, reg) | Write 1-byte reg pointer, read 1 byte back |
| i2cWriteReg8(addr, reg, val) | Write reg + val in one transmission |
| i2cReadRegs(addr, reg, buf, len) | Burst read len bytes starting at reg |

## 4.2  DrvMpu6500.h/.cpp

The MPU6500 driver auto-detects the I2C address (0x68 or 0x69) by reading WHO_AM_I. Valid responses are 0x70 or 0x71. If neither address responds correctly, beginAuto() returns false and the application transitions to ERROR.

Initialization sequence:

- Wake the device and select PLL clock source (PWR_MGMT_1 = 0x01). Delay 100 ms.

- Set sample rate divider to achieve 200 Hz raw output (SMPLRT_DIV = 9 for 200 Hz from 2 kHz internal).

- Configure DLPF: REG_CONFIG = 0x03 (44 Hz accel, 42 Hz gyro bandwidth). Reduces noise without phase lag issues at 100 Hz filter rate.

- Gyro range: REG_GYRO_CONFIG = 0x00 (±250 dps, scale = 131.0 LSB/dps).

- Accel range: REG_ACCEL_CONFIG = 0x00 (±2 g, scale = 16384.0 LSB/g).

readAccelGyro() reads 14 bytes starting at ACCEL_XOUT_H in a single burst. Bytes 0-5 are accel XYZ, bytes 6-7 are temperature (discard), bytes 8-13 are gyro XYZ. All values are big-endian int16. Divide by scale constants to get g and dps.

Output units: accel in g (float), gyro in dps (float). Conversion to rad/s happens at the calibration stage, not here. Keep driver outputs in natural sensor units.

## 4.3  DrvTlv493d.h/.cpp

The TLV493D has no reset pin. Use the I2C general-call reset (address 0x00, write byte 0x06) after power-up. The device latches its I2C address based on the SDA line state at startup; both 0x1F and 0x5E are possible. The driver probes both.

Bring-up sequence (up to 4 attempts):

- Issue general-call reset. Delay 10 ms.

- Probe 0x1F, then 0x5E.

- If a device ACKs, call dev.begin() then dev.setAccessMode(FASTMODE).

- Run sanity check: read twice 20 ms apart. Reject if data is all-zero, frame counter does not change, or both readings are identical.

- If sanity check fails, retry from step 1. If all 4 attempts fail, return false.

readmT() calls dev.updateData() then checks the 2-bit frame counter. If the counter matches the last seen value, the sample is stale and readmT() returns false without updating the output. This prevents duplicate samples from entering the filter.

Output units: microtesla (mT) as float. The TLV493D returns mT natively. Do not convert units here.

# 5. Calibration System

## 5.1  NVS Storage (NvsCal.h/.cpp)

All calibration data persists in ESP32 NVS under namespace 'orient_cal', key 'cal_blob'. The blob is versioned and CRC32-protected.

CalBlob layout:

| Field | Type | Description |
| --- | --- | --- |
| magic | uint32_t | 0xCA11BA11 — validity check |
| version | uint16_t | Schema version; reject mismatches |
| size | uint16_t | sizeof(CalBlob) — size guard |
| crc32 | uint32_t | CRC32 over entire blob with this field = 0 |
| mag_valid | uint8_t | 1 = mag calibration data is usable |
| accel_valid | uint8_t | 1 = accel bias data is usable |
| gyro_valid | uint8_t | 1 = gyro bias data is usable |
| mag_offset[3] | float[3] | Hard iron offset (mT), XYZ |
| mag_softmat[9] | float[9] | 3x3 soft iron correction matrix, row-major |
| accel_bias[3] | float[3] | Accel bias (g), XYZ |
| gyro_bias[3] | float[3] | Gyro bias (rad/s), XYZ |
| saved_at_ms | uint32_t | millis() at last save |

calSave(): compute CRC32 over blob with crc32 field zeroed. Write magic, version, size, then CRC. Store via Preferences.putBytes(). Return false if write byte count does not equal sizeof(CalBlob).

calLoad(): read blob. Verify magic, version, and size. Recompute CRC and compare. Only populate g_cal if all checks pass. If loading fails for any reason, call calResetDefaults() to ensure a clean known state rather than partially populated fields.

## 5.2  Magnetometer Calibration (CalMag.h/.cpp)

Mag calibration corrects two types of error:

- Hard iron: constant offset caused by nearby ferromagnetic material. Appears as a translation of the measurement sphere. Removed by subtracting a 3D offset vector.

- Soft iron: directional scaling caused by material that distorts the field. Appears as the sphere becoming an ellipsoid. Corrected by multiplying by a 3x3 matrix.

### 5.2.1  Sphere Coverage Metric

The unit sphere is divided into 72 bins: 12 azimuth sectors (30 deg each) × 6 elevation bands (-90 to +90 deg, 30 deg each). For each incoming magnetometer sample, compute its azimuth and elevation, map to a bin index, and mark that bin as filled.

az  = atan2f(my, mx) * RAD_TO_DEG  // 0-360 after normalizing

el  = asinf(mz / norm) * RAD_TO_DEG  // -90 to +90

bin = (az_bucket * 6) + el_bucket    // 0-71

Report coverage as filled_bins / 72 * 100. Serial output every 25 samples:

CAL_PROG samples=143 coverage=61% next_axis=z

Do not attempt the ellipsoid fit until both conditions are met: sample count >= kMagMinSamples AND coverage >= kMagMinCoveragePct. If CMD:CAL_MAG_STOP arrives before these thresholds, respond with ERR and remain in calibrating state.

### 5.2.2  Ellipsoid Fit (Hard + Soft Iron)

With sufficient coverage, fit an ellipsoid to the raw magnetometer point cloud. The algebraic approach fits the equation xT A x + bT x + c = 0 to the data. For embedded use, the simplified min/max per-axis approach is acceptable for hard iron, but full ellipsoid fitting is required to get the off-diagonal soft iron terms.

Implementation approach for ESP32 (no dynamic allocation):

- Accumulate the 6 unique products (xx, yy, zz, xy, xz, yz) and the 3 linear terms (x, y, z) as double-precision running sums. Store up to kMagMinSamples * 2 raw samples in a static ring buffer.

- On CAL_STOP, form the 9x9 design matrix D from the accumulated sums.

- Solve for the ellipsoid coefficients using Cholesky decomposition or a fixed-size Gauss-Jordan elimination (no heap needed for 9x9).

- Extract the center (hard iron offset) and the shape matrix (soft iron). The soft iron correction matrix is the inverse of the ellipsoid's shape matrix, scaled to normalize the sphere.

- Write the 3-element offset and 9-element matrix to g_cal. Call calSave().

If the matrix is singular or the condition number is too high (coverage was insufficient despite the thresholds), the fit fails gracefully. Return an error, keep the old calibration in NVS, and report the failure reason on serial.

### 5.2.3  Applying Calibration

// Remove hard iron

float cx = mx - mag_offset[0];

float cy = my - mag_offset[1];

float cz = mz - mag_offset[2];

// Apply 3x3 soft iron matrix (row-major, mag_softmat[row*3+col])

float fx = softmat[0]*cx + softmat[1]*cy + softmat[2]*cz;

float fy = softmat[3]*cx + softmat[4]*cy + softmat[5]*cz;

float fz = softmat[6]*cx + softmat[7]*cy + softmat[8]*cz;

This runs on every raw magnetometer sample before the value enters the Madgwick filter.

## 5.3  IMU Calibration (CalImu.h/.cpp)

### 5.3.1  Startup Gyro Bias

On power-up, before the main loop starts, collect a blocking 3-second sample at 10 ms intervals. Accept only samples where:

- Accel norm is within 0.88 g – 1.12 g (device is roughly level and still)

- Gyro norm is below 8 dps (not being moved during boot)

Average the accepted gyro samples (in rad/s) across all three axes. Store in g_cal.gyro_bias. Require a minimum of 50 accepted samples before trusting the result. If the device was being moved during boot and fewer than 50 samples were accepted, gyro_valid = 0. The filter will still run but yaw will drift until auto-recal fires.

### 5.3.2  Static Detection

GyroBiasLearner maintains a ring buffer of the last kGyroStillRingN samples (default 200, covering 2 seconds at 100 Hz). For each incoming sample, compute:

- Accel norm mean and std-dev

- Gyro norm std-dev

- Per-axis gyro std-dev (X, Y, Z separately)

The device is considered still when ALL of the following pass:

| Condition | Threshold | Rationale |
| --- | --- | --- |
| Accel norm mean | 0.88 – 1.12 g | Confirms gravity vector present |
| Accel norm std-dev | < 0.03 g | No vibration or shock |
| Gyro norm std-dev | < 0.01 rad/s | No rotation noise |
| Each axis gyro std-dev | < 0.01 rad/s | Per-axis sensitivity |

A 'streak' counter increments when still conditions pass, resets to zero on any fail. The device is declared still only when the streak reaches a minimum of 20 consecutive passing windows.

### 5.3.3  Auto-Recalibration

When the device has been still (streak threshold met) AND kAutoRecalIntervalMs has elapsed since the last successful recal, update the gyro bias using an IIR filter:

bias[i] = (1.0f - ALPHA) * bias[i] + ALPHA * raw_gyro_rs[i]

ALPHA = 0.02f (slow IIR, prevents a single noisy sample from corrupting the bias estimate). Update g_cal.gyro_bias and set gyro_valid = 1. Reset the interval timer. Do not call calSave() on every auto-recal; save only on explicit CMD:SAVE_CAL to avoid NVS write wear.

Accel bias follows the same static detection gate. When still, compute the difference between measured accel and expected gravity vector [0, 0, 1] in body frame. Update accel_bias with the same IIR. Apply accel bias correction before the Madgwick update.

# 6. Madgwick Filter (fusion/Madgwick.h/.cpp)

## 6.1  What the filter does

The Madgwick filter maintains a quaternion estimate of orientation. At each step it integrates the gyro rate, then applies a gradient-descent correction term derived from the accelerometer (gravity reference for pitch/roll) and the magnetometer (magnetic north reference for yaw). The correction strength is controlled by beta.

## 6.2  Input requirements

| Input | Units | Notes |
| --- | --- | --- |
| gx, gy, gz | rad/s | Gyro after bias removal |
| ax, ay, az | g (or m/s²) | Accel after bias removal; filter normalizes internally |
| mx, my, mz | any consistent unit | Mag after hard/soft iron correction; filter normalizes internally |
| beta | dimensionless | Pass kBetaStill or kBetaMove based on stillness detector |
| dt | seconds | Actual elapsed time since last update; compute from millis() |

## 6.3  Update function structure

The filter exposes two update paths:

- update(gx, gy, gz, ax, ay, az, mx, my, mz, beta, dt) — full AHRS with magnetometer

- updateIMU(gx, gy, gz, ax, ay, az, beta, dt) — fallback used when mag norm < 1e-6 (sensor absent or reading zero)

In the main loop, always call update(). The function internally detects a zero mag vector and falls back to updateIMU(). This means yaw will drift if the magnetometer fails, but pitch and roll remain stable.

## 6.4  Quaternion normalization

Normalize the quaternion at the end of every update. Check the norm before dividing; if it is below 1e-9, the quaternion has degenerated (this should not happen in normal operation but guard against it anyway). Do not normalize in the middle of the gradient computation — only at the very end.

## 6.5  Euler conversion

Call eulerDeg() only when building the output packet, not inside the filter loop. ZYX order:

roll  = atan2f(2*(q0*q1 + q2*q3), 1 - 2*(q1*q1 + q2*q2))

pitch = asinf(clamp(2*(q0*q2 - q3*q1), -1, 1))

yaw   = atan2f(2*(q0*q3 + q1*q2), 1 - 2*(q2*q2 + q3*q3))

After conversion to degrees, apply declination correction to yaw:

yaw -= kMagDeclinationDeg;

if (yaw < 0.0f)   yaw += 360.0f;

if (yaw >= 360.0f) yaw -= 360.0f;

## 6.6  Dynamic beta selection

Pass the beta value in as a parameter rather than storing it inside the filter class. The caller (Application) decides the beta based on the current stillness detection state. This keeps the filter class stateless with respect to beta, making it easier to test.

# 7. Communications Layer

## 7.1  Binary Packet Format

Every orientation output is a 17-byte binary packet. The format is fixed and must not change without incrementing a protocol version. Multi-byte fields are little-endian.

| Byte(s) | Field | Type | Description |
| --- | --- | --- | --- |
| 0 | START | uint8 | Fixed 0xAA |
| 1 | SEQ | uint8 | Sequence number, wraps 0-255 |
| 2-5 | YAW | float32 | Degrees, 0.0 – 360.0 |
| 6-9 | PITCH | float32 | Degrees, -90.0 – +90.0 |
| 10-13 | ROLL | float32 | Degrees, -180.0 – +180.0 |
| 14 | FLAGS | uint8 | Status flags (see below) |
| 15-16 | CRC16 | uint16 | CRC-16/CCITT-FALSE over bytes 0-14 |

### 7.1.1  Status Flags Byte

| Bit | Name | Meaning when set (1) |
| --- | --- | --- |
| 0 | MAG_CAL_VALID | Magnetometer calibration loaded and applied |
| 1 | IMU_CAL_VALID | Gyro and accel biases loaded and applied |
| 2 | FILTER_CONVERGED | Filter has run for at least 2 seconds post-init |
| 3 | STATIC | Static detection streak active (device is still) |
| 4-7 | RESERVED | Set to 0 |

### 7.1.2  CRC-16 Computation

Use CRC-16/CCITT-FALSE (polynomial 0x1021, initial value 0xFFFF, no input/output reflection). Compute over bytes 0-14 inclusive. Store result little-endian in bytes 15-16.

uint16_t crc16(const uint8_t* data, size_t len) {

uint16_t crc = 0xFFFF;

for (size_t i = 0; i < len; i++) {

crc ^= (uint16_t)data[i] << 8;

for (int b = 0; b < 8; b++)

crc = (crc & 0x8000) ? (crc << 1) ^ 0x1021 : crc << 1;

}

return crc;

}

## 7.2  READY Handshake

After setup() completes successfully (sensors init, NVS load attempted, state reaches IDLE or READY), send exactly:

READY\n

The host must not send any commands or expect any packets until it receives this string. If setup() fails, the ESP32 sends ERROR:<reason>\n instead and never sends READY. The host must time out after kReadyTimeoutMs (10 s) and report a connection error.

## 7.3  Telemetry Command Set (ASCII)

Commands arrive as ASCII lines terminated with \n. Each command gets exactly one response line before the next command is accepted.

| Command | Response | Behaviour |
| --- | --- | --- |
| CMD:STATUS | OK state=... cal=... | Current state machine state and cal flags |
| CMD:CAL_MAG | OK / ERR | Start magnetometer calibration collection |
| CMD:CAL_MAG_STOP | OK coverage=% / ERR | Stop collection and attempt ellipsoid fit |
| CMD:CAL_IMU | OK / ERR | Trigger immediate startup gyro/accel bias collection |
| CMD:SAVE_CAL | OK / ERR | Write current g_cal to NVS |
| CMD:LOAD_CAL | OK / ERR | Reload g_cal from NVS (hot reload) |
| CMD:CLEAR_CAL | OK | Erase NVS and reset to defaults |
| CMD:RESET | (reboots) | esp_restart() |

During CMD:CAL_MAG collection, binary orientation packets continue streaming. The host must handle interleaved binary packets and ASCII calibration progress lines. Progress lines are always ASCII and start with 'CAL_PROG'.

## 7.4  UartTransport

The UART layer uses a software ring buffer for RX. It reads up to kUartReadBudgetBytesPerLoop bytes per loop() call to bound CPU time. Lines are accumulated until \n is seen or kLineTimeoutMs elapses, whichever comes first. On timeout the partial line is discarded.

The transmit path is direct Serial.write(); no TX ring buffer is needed because packet writes are short (17 bytes) and fit in the hardware FIFO. Do not use Serial.print(float) anywhere — it silently uses double arithmetic. Always format floats explicitly with snprintf() for ASCII responses.

# 8. Application Loop

## 8.1  Timing Architecture

All scheduling is cooperative, no RTOS. Three time-sliced tasks run from loop():

| Task | Period | Work done |
| --- | --- | --- |
| IMU read | 5 ms (200 Hz) | Read MPU6500 + TLV493D, apply calibration, push to sample buffer |
| Filter + output | 10 ms (100 Hz) | Run Madgwick, build packet, write to serial |
| UART service | Every loop() | Drain RX ring, parse commands, dispatch responses |

The IMU runs at 200 Hz to oversample the MPU6500. The filter runs at 100 Hz. When the filter fires, it uses the most recent IMU sample. This means accel/gyro data is always at most 5 ms old when it enters the filter, which is acceptable at 100 Hz output.

## 8.2  Main Loop Pseudocode

void Application::loopOnce() {

uint32_t now = clock_.millis();

uart_.service(now);           // always first

processCommands(now);

if (elapsed(now, last_imu_ms, 5))  { readSensors(now); }

if (elapsed(now, last_filt_ms, 10)) { runFilter(now);  }

}

## 8.3  Sensor Read Pipeline

- Call mpu.readAccelGyro() — get raw accel (g) and gyro (dps).

- Subtract g_cal.gyro_bias (rad/s). Convert gyro from dps to rad/s first.

- Subtract g_cal.accel_bias. If accel_valid = 0, skip subtraction.

- Push sample to GyroBiasLearner.pushSample(). Update stillness detection.

- Call tlv.readmT() — get raw mag (mT). Returns false on stale frame.

- If mag read succeeded, apply magApplyCal() (hard/soft iron correction).

- Store corrected values in latest_sample struct.

## 8.4  Filter and Packet Pipeline

- Compute dt = (now - last_filter_ms) / 1000.0f.

- Select beta: kBetaStill if gyroBiasLearner.isStill(), else kBetaMove.

- Call madgwick.update() with corrected sensor values, beta, dt.

- Increment filter_ticks counter. Set FILTER_CONVERGED flag when ticks > (kFilterHz * 2).

- Call madgwick.eulerDeg() to get yaw/pitch/roll.

- Build 17-byte packet. Compute CRC16. Write to serial via Serial.write().

# 9. Host Python Package

The host runs on the Jetson Orin Nano. It is a Python 3.10+ package in host/orientation_host/. Dependencies: pyserial only. No external libraries beyond stdlib.

## 9.1  File Layout

host/

orientation_host/

__init__.py

serial_handler.py    # connection, READY wait, RX threading

packet.py            # parse 17-byte packets, CRC16 validation

cli.py               # interactive command-line interface

tests/

test_packet.py       # parse known-good and known-bad packets

test_crc.py          # CRC16 implementation cross-check

requirements.txt       # pyserial

## 9.2  serial_handler.py

SerialHandler opens the port, then blocks waiting for the 'READY\n' string from the ESP32. If READY is not received within kReadyTimeoutMs, raise ConnectionError. After READY, start a background daemon thread that reads bytes continuously and feeds them into a shared buffer.

The background thread scans the buffer for the 0xAA start byte. When found, it reads the next 16 bytes and passes the 17-byte packet to PacketParser. If the start byte is not 0xAA, discard the byte and continue scanning. This handles any framing loss without getting stuck.

ASCII lines (calibration progress, command responses) are also emitted by the ESP32 between packets. Any byte sequence not starting with 0xAA and ending with 0x0A (newline) is treated as an ASCII line and emitted to a separate ASCII queue. The CLI drains both queues independently.

## 9.3  packet.py

parse_packet(data: bytes) -> dict | None

Accepts exactly 17 bytes. Returns None if:

- Byte 0 is not 0xAA

- CRC16 of bytes 0-14 does not match bytes 15-16 (little-endian uint16)

On success returns a dict with keys: seq (int), yaw (float), pitch (float), roll (float), flags (int), and individual flag booleans mag_cal_valid, imu_cal_valid, filter_converged, is_static.

Float parsing uses struct.unpack('<f', ...) for each 4-byte field. Use '<' (little-endian) consistently. The CRC16 implementation in Python must match the ESP32 exactly — test this with a known packet before trusting live data.

## 9.4  cli.py

A simple interactive loop. On start: connect, wait for READY, print confirmation with firmware version from CMD:STATUS. Then enter a display loop printing live yaw/pitch/roll at 10 Hz from the latest parsed packet, and accept single-line commands typed by the user.

Commands are sent as-is to the serial port with a \n appended. The response (ASCII line from the ESP32) is printed before the display resumes. A clean KeyboardInterrupt exits without error.

# 10. Testing and Validation Gates

Each build phase has a specific terminal-observable pass criterion. Claude Code reads the serial monitor output after each upload and must confirm the pass criterion before proceeding to the next phase.

| Phase | Pass criterion | What to check in serial output |
| --- | --- | --- |
| 1 | Sensor reads | Non-zero values on all 6 axes. MPU: accel z ≈ 1.0g on flat surface. Mag: all three axes non-zero and changing when rotated. |
| 2 | Filter output | READY received. Flat on table: pitch < 2 deg, roll < 2 deg. Yaw changes when rotated. No NaN or Inf in any field. |
| 3 | Binary packets | Python test script reads 20 packets, all pass CRC validation, sequence numbers increment without gaps. |
| 4 | Command parser | CMD:STATUS returns OK with state=IDLE or READY. CMD:PING returns OK. Unknown command returns ERR. |
| 5 | Mag calibration | CAL_PROG lines appear during tumble. Coverage reaches >= 60% before fit. calSave() returns true. After reboot, calLoad() restores values. |
| 6 | Gyro auto-recal | Hold still 3 seconds: serial shows static detection active. Bias values visible in CMD:STATUS output. |
| 7 | Full integration | All status flags correct. Yaw stable pointing North. Roll and pitch stable on flat surface. CLI shows live orientation. |

## 10.1  Known Invariants (never break these)

- Packet start byte is always 0xAA. Never write any packet without it.

- CRC16 is computed over bytes 0-14. Never include the CRC bytes themselves in the CRC computation.

- Quaternion norm is checked after every filter step. A zero-norm quaternion must be caught and logged, not silently propagated.

- NVS save is never called from the filter loop. Only called from command handlers or calSave() explicitly.

- Serial.print(float) is never used. All float formatting goes through snprintf().

- Double precision is never used in the filter or calibration math. All filter variables are float. -Wdouble-promotion will catch violations at compile time.

# 11. CLAUDE.md (copy verbatim into project root)

This file is read by Claude Code at the start of every session. It must be present in the project root before any build prompt is issued.

# Orientation System — Build Context

## Hardware

ESP32 (PlatformIO, Arduino framework)

MPU6500 accel+gyro, I2C address 0x68 or 0x69 (auto-detect)

TLV493D magnetometer, I2C address 0x1F or 0x5E (auto-detect)

Host: Jetson Orin Nano, Python 3.10+

UART: 115200 baud, /dev/ttyUSB0

## Coordinate Frame

ENU: X=East, Y=North, Z=Up

Both sensors physically aligned to ENU — NO axis remapping needed

Yaw: 0-360 deg, clockwise from North (compass bearing)

Pitch: -90 to +90 deg, Roll: -180 to +180 deg

Internal: quaternion float32. Output: Euler float32, ZYX order

## Packet Format (17 bytes, little-endian)

0: 0xAA start | 1: seq uint8 | 2-5: yaw f32 | 6-9: pitch f32

10-13: roll f32 | 14: flags uint8 | 15-16: CRC16 uint16

CRC16/CCITT-FALSE over bytes 0-14. Poly=0x1021 Init=0xFFFF

## Commands (ASCII, newline terminated)

CMD:STATUS CMD:CAL_MAG CMD:CAL_MAG_STOP CMD:CAL_IMU

CMD:SAVE_CAL CMD:LOAD_CAL CMD:CLEAR_CAL CMD:RESET

## Handshake

ESP32 sends READY\n after successful init

Host waits up to 10 seconds for READY before timeout error

## Calibration

Mag: ellipsoid fit, 72-bin sphere coverage, save 3x1 offset + 3x3 matrix to NVS

Gyro: startup blocking cal + IIR auto-recal when static (gyro std < 0.01 rad/s)

Accel: IIR bias update when static, gravity vector reference

NVS magic: 0xCA11BA11, version: 1, namespace: orient_cal, key: cal_blob

Declination: -0.8 deg (Bengaluru, hardcoded constant kMagDeclinationDeg)

## Compile rules

-Werror -Wdouble-promotion: no double in filter math, ever

No Serial.print(float): use snprintf() for all float formatting

No magic numbers: all constants in core/Config.h

## Validation

After upload: read serial monitor, confirm READY, check pass criteria from spec

Phase 1: non-zero sensor reads on all axes

Phase 2: pitch<2deg roll<2deg flat, yaw changes on rotation

Phase 3: 20 packets all CRC pass, seq increments

Phase 4: CMD:STATUS returns OK, unknown cmd returns ERR

Phase 5: coverage>=60% before fit, NVS survives reboot

Phase 6: static detection visible in STATUS after 3s hold-still

Phase 7: all flags correct, yaw stable North, live CLI works

End of specification — Rev 1.0